# Experiment-2

2. Design and Performance Analysis of Chebyshev Type-I IIR Low-Pass and High-Pass Filters Using MATLAB

CO Mapping:CO1

Blooms Level K4-Analyze

WK Level :WK3 Engineering Fundamentals (Analyzes ripple, attenuation, order and frequency response)

➔ Major Topic

Chebyshev IIR Filters

➔ Minor Topics

Passband ripple, filter order, frequency response, comparison with Butterworth

➔ Objectives of this Experiment:

1. To design Chebyshev Type-I IIR low-pass and high-pass filters for specified digital frequency requirements.

2. To study the effect of passband ripple on filter order and frequency response.

3. To obtain and analyze filter coefficients, magnitude and phase responses.

4. To compare the Chebyshev Type-I response with the corresponding Butterworth response.

➔ Expected Outcome from this Experiment:

Students shall be able to:

1. Design Chebyshev Type-I low-pass and high-pass filters and obtain the corresponding filter coefficients.

2. Determine the required filter order and verify the specified passband ripple.

3. Generate and interpret magnitude and phase responses and determine the usable bandwidth.

4. Compare Chebyshev and Butterworth responses in terms of ripple, transition behavior and order.

4. Compare Chebyshev Type-I and Butterworth responses in terms of ripple, transition behavior and filter order.

MATLAB with Signal Processing Toolbox.

➔ Objective linkage:

The experiment is structured so that each objective directly produces a verifiable output listed in the Output Parameter Table.

➔ Experimental Procedure:

1. Define sampling frequency, passband/stopband edges and allowable passband ripple.

2. Normalize the digital frequency specifications.

3. Determine the minimum Chebyshev Type-I order and cutoff frequency.

4. Generate low-pass and high-pass coefficients and plot their frequency responses.

5. Measure/identify passband ripple, transition region, cutoff and stopband behavior.

6. Design a corresponding Butterworth filter using the same broad specifications and compare the responses.

➔ INPUT PARAMETER TABLE – VALUES TO BE ENTERED

| S.No | Input Parameter | Symbol / Setting | Value to Enter | Unit |
| --- | --- | --- | --- | --- |
| 1 | Sampling frequency | Fs | 1000 | Hz |
| 2 | Passband edge | Fp | 100 | Hz |
| 3 | Stopband edge | Fst | 150 | Hz |
| 4 | Passband ripple | Rp | 1 | dB |
| 5 | Stopband attenuation | Rs | 40 | dB |

%% Experiment 2: Chebyshev Type-I IIR Low-Pass and High-Pass Filters

clc; clear; close all;

%% 1. Input parameters

Fs  = 1000;          % Sampling frequency (Hz)

Rp  = 1;             % Passband ripple (dB)

Rs  = 40;            % Stopband attenuation (dB)

% Low-pass edges

Fp_LP  = 100;  Fst_LP = 150;

% High-pass edges (mirror of the LPF: stopband below, passband above)

Fst_HP = 100;  Fp_HP  = 150;

% (To reuse Experiment 1 values, set Fst_HP = 200 and Fp_HP = 250.)

%% 2. Normalize frequencies (w.r.t. Nyquist = Fs/2)

Wp_LP = Fp_LP /(Fs/2);   Ws_LP = Fst_LP/(Fs/2);

Wp_HP = Fp_HP /(Fs/2);   Ws_HP = Fst_HP/(Fs/2);

%% 3. Order and cutoff: Chebyshev Type-I

[N_C_LP, Wn_C_LP] = cheb1ord(Wp_LP, Ws_LP, Rp, Rs);

[N_C_HP, Wn_C_HP] = cheb1ord(Wp_HP, Ws_HP, Rp, Rs);

%% 4. Chebyshev Type-I coefficients

[bC_LP, aC_LP] = cheby1(N_C_LP, Rp, Wn_C_LP, 'low');

[bC_HP, aC_HP] = cheby1(N_C_HP, Rp, Wn_C_HP, 'high');

fprintf('--- Chebyshev Type-I Low-Pass ---\n');

fprintf('Order N = %d, Wn = %.4f, passband edge = %.2f Hz\n', N_C_LP, Wn_C_LP, Wn_C_LP*Fs/2);

disp('Numerator b ='); disp(bC_LP);

disp('Denominator a ='); disp(aC_LP);

fprintf('--- Chebyshev Type-I High-Pass ---\n');

fprintf('Order N = %d, Wn = %.4f, passband edge = %.2f Hz\n', N_C_HP, Wn_C_HP, Wn_C_HP*Fs/2);

disp('Numerator b ='); disp(bC_HP);

disp('Denominator a ='); disp(aC_HP);

%% 5. Butterworth filters with the same specifications (for comparison)

[N_B_LP, Wn_B_LP] = buttord(Wp_LP, Ws_LP, Rp, Rs);

[N_B_HP, Wn_B_HP] = buttord(Wp_HP, Ws_HP, Rp, Rs);

[bB_LP, aB_LP] = butter(N_B_LP, Wn_B_LP, 'low');

[bB_HP, aB_HP] = butter(N_B_HP, Wn_B_HP, 'high');

%% 6. Frequency responses

[HC_LP, f] = freqz(bC_LP, aC_LP, 2048, Fs);

[HC_HP, ~] = freqz(bC_HP, aC_HP, 2048, Fs);

[HB_LP, ~] = freqz(bB_LP, aB_LP, 2048, Fs);

[HB_HP, ~] = freqz(bB_HP, aB_HP, 2048, Fs);

%% 7. Magnitude response of Chebyshev filters

figure;

subplot(2,1,1);

plot(f, 20*log10(abs(HC_LP)), 'b', 'LineWidth', 1.5); grid on; hold on;

yline(-Rp, '--g'); yline(-Rs, '--r');

xline(Fp_LP, '--g'); xline(Fst_LP, '--r');

title(sprintf('Chebyshev Type-I LPF Magnitude Response (N = %d)', N_C_LP));

xlabel('Frequency (Hz)'); ylabel('Magnitude (dB)'); ylim([-100 5]);

subplot(2,1,2);

plot(f, 20*log10(abs(HC_HP)), 'r', 'LineWidth', 1.5); grid on; hold on;

yline(-Rp, '--g'); yline(-Rs, '--r');

xline(Fst_HP, '--r'); xline(Fp_HP, '--g');

title(sprintf('Chebyshev Type-I HPF Magnitude Response (N = %d)', N_C_HP));

xlabel('Frequency (Hz)'); ylabel('Magnitude (dB)'); ylim([-100 5]);

%% 8. Phase response of Chebyshev filters

figure;

subplot(2,1,1);

plot(f, unwrap(angle(HC_LP))*180/pi, 'b', 'LineWidth', 1.5); grid on;

title('Chebyshev Type-I LPF Phase Response');

xlabel('Frequency (Hz)'); ylabel('Phase (degrees)');

subplot(2,1,2);

plot(f, unwrap(angle(HC_HP))*180/pi, 'r', 'LineWidth', 1.5); grid on;

title('Chebyshev Type-I HPF Phase Response');

xlabel('Frequency (Hz)'); ylabel('Phase (degrees)');

%% 9. Passband ripple measurement and verification

magC_LP = 20*log10(abs(HC_LP));  magC_HP = 20*log10(abs(HC_HP));

pb_LP = f <= Fp_LP;              pb_HP = f >= Fp_HP;

ripple_LP = max(magC_LP(pb_LP)) - min(magC_LP(pb_LP));

ripple_HP = max(magC_HP(pb_HP)) - min(magC_HP(pb_HP));

mag_dB = @(b,a,fx) 20*log10(abs(freqz(b,a,fx,Fs)));

fprintf('\n--- Verification (Chebyshev) ---\n');

fprintf('LPF measured passband ripple = %.3f dB (spec %.1f dB)\n', ripple_LP, Rp);

fprintf('HPF measured passband ripple = %.3f dB (spec %.1f dB)\n', ripple_HP, Rp);

fprintf('LPF gain at %d Hz (passband edge): %.3f dB\n', Fp_LP,  mag_dB(bC_LP,aC_LP,Fp_LP));

fprintf('LPF gain at %d Hz (stopband edge): %.3f dB (need <= -%d)\n', Fst_LP, mag_dB(bC_LP,aC_LP,Fst_LP), Rs);

fprintf('HPF gain at %d Hz (stopband edge): %.3f dB (need <= -%d)\n', Fst_HP, mag_dB(bC_HP,aC_HP,Fst_HP), Rs);

fprintf('HPF gain at %d Hz (passband edge): %.3f dB\n', Fp_HP,  mag_dB(bC_HP,aC_HP,Fp_HP));

% Usable bandwidth (frequencies where gain stays within Rp dB of 0 dB)

fprintf('LPF usable bandwidth: 0 to %.2f Hz\n', Fp_LP);

fprintf('HPF usable bandwidth: %.2f Hz to %.2f Hz\n', Fp_HP, Fs/2);

%% 10. Chebyshev vs Butterworth comparison

figure;

subplot(2,1,1);

plot(f, 20*log10(abs(HC_LP)), 'b', f, 20*log10(abs(HB_LP)), 'r--', 'LineWidth', 1.5);

grid on; hold on; xline(Fp_LP,':g'); xline(Fst_LP,':r');

legend(sprintf('Chebyshev I (N=%d)', N_C_LP), sprintf('Butterworth (N=%d)', N_B_LP));

title('LPF: Chebyshev Type-I vs Butterworth');

xlabel('Frequency (Hz)'); ylabel('Magnitude (dB)'); ylim([-100 5]);

subplot(2,1,2);

plot(f, 20*log10(abs(HC_HP)), 'b', f, 20*log10(abs(HB_HP)), 'r--', 'LineWidth', 1.5);

grid on; hold on; xline(Fst_HP,':r'); xline(Fp_HP,':g');

legend(sprintf('Chebyshev I (N=%d)', N_C_HP), sprintf('Butterworth (N=%d)', N_B_HP));

title('HPF: Chebyshev Type-I vs Butterworth');

xlabel('Frequency (Hz)'); ylabel('Magnitude (dB)'); ylim([-100 5]);

% Passband zoom to show ripple

figure;

plot(f, 20*log10(abs(HC_LP)), 'b', f, 20*log10(abs(HB_LP)), 'r--', 'LineWidth', 1.5);

grid on; xlim([0 Fst_LP]); ylim([-3 0.5]);

legend('Chebyshev I', 'Butterworth');

title('LPF Passband Zoom: Equiripple (Chebyshev) vs Maximally Flat (Butterworth)');

xlabel('Frequency (Hz)'); ylabel('Magnitude (dB)');

%% 11. Effect of passband ripple on filter order

Rp_list = [0.1 0.5 1 2 3];

N_cheb = zeros(size(Rp_list));  N_butt = zeros(size(Rp_list));

for k = 1:numel(Rp_list)

N_cheb(k) = cheb1ord(Wp_LP, Ws_LP, Rp_list(k), Rs);

N_butt(k) = buttord (Wp_LP, Ws_LP, Rp_list(k), Rs);

end

fprintf('\n--- Effect of Rp on LPF order ---\n');

fprintf('Rp (dB)   N_Cheby1   N_Butter\n');

for k = 1:numel(Rp_list)

fprintf('%5.1f     %5d      %5d\n', Rp_list(k), N_cheb(k), N_butt(k));

end

figure;

stem(Rp_list, N_cheb, 'b', 'filled'); hold on;

stem(Rp_list, N_butt, 'r', 'filled'); grid on;

legend('Chebyshev I', 'Butterworth');

xlabel('Passband ripple Rp (dB)'); ylabel('Filter order N');

title('Filter Order vs Passband Ripple');

%% 12. Pole-zero plots and stability

figure;

subplot(1,2,1); zplane(bC_LP, aC_LP); title('Chebyshev LPF Pole-Zero'); grid on;

subplot(1,2,2); zplane(bC_HP, aC_HP); title('Chebyshev HPF Pole-Zero'); grid on;

fprintf('\nChebyshev LPF stable? %d (max |pole| = %.4f)\n', all(abs(roots(aC_LP))<1), max(abs(roots(aC_LP))));

fprintf('Chebyshev HPF stable? %d (max |pole| = %.4f)\n', all(abs(roots(aC_HP))<1), max(abs(roots(aC_HP))));

%% 13. Comparison summary table

fprintf('\n--- Comparison Summary (LPF) ---\n');

fprintf('Filter          Order   Passband ripple\n');

fprintf('Chebyshev I     %5d   %.3f dB (equiripple)\n', N_C_LP, ripple_LP);

rippleB = max(20*log10(abs(HB_LP(pb_LP)))) - min(20*log10(abs(HB_LP(pb_LP))));

fprintf('Butterworth     %5d   %.3f dB (monotonic)\n', N_B_LP, rippleB);